| M-4-97 Dawson Bay N. Mafeking Quarry SC-1 50° angle | 7-32-44-25W 5855308N 361965E 279 m | OVERBURDEN DEVONIAN/Souris River/(Point Wilkins) | 0.0-1.7 1.7-3.0 3.0-9.3 | Rubbly limestone Dolomitic Limestone Beds: rusty brown; limestone to dolomitic limestone; mudstone; some intense red colouration Micritic Limestone Beds: mottled limestone; mudstone to wackestone; fossiliferous |
| --- | --- | --- | --- | --- |
| M-5-97 Dawson Bay N. Mafeking Quarry SC-1 | 8-33-44-25W 5855302N 363929E 274 m | OVERBURDEN DEVONIAN/Souris River (Point Wilkins) | 0.0-3.8 3.8-6.9 6.9-8.2 8.2-22.3 | Boulder till; possible silica sinter pebble at the base Micritic Limestone Beds: tan limestone; wackestone; fossiliferous Sand clay infill; brown sand; some green grey clay; silica sinter pebble at the base Micritic Limestone Beds: as above; very broken |
| M-6-97 North Dawson Bay N. Mafeking Quarry | 7-32-44-25W 5855302N 561872E 279 m | DEVONIAN/Souris River (Point Wilkins) (First Red Beds) Dawson Bay/(Upper) Dawson Bay (Middle) Dawson Bay (Lower) (Second Red Beds) | 0.0-0.6 0.6-25.5 25.5-34.3 34.3-44.7 44.7-51.6 51.6-69.4 69.4-76.2 76.2-81.4 | Dolomitic Limestone Beds: yellow orange to tan; limestone; wackestone Micritic Limestone Beds: light tan to brown; mottled limestone; fossiliferous Argillaceous Limestone Beds: light brown tan; limestone; mudstone; scattered light green clay Red brown to green limestone; mudstone Limestone: wackestone; light tan to dark brown; abundant green grey clay along fractures and bedding planes Limestone: mudstone; dark grey to light olive green Limestone: wackestone; light tan; fossiliferous Limestone: grey to dark red argillaceous mudstone |
| M-7-97 Dawson Bay Borrow Pit East of Mafeking Quarry | 12-33-44-25W 5855935N 362619E 276 m | OVERBURDEN DEVONIAN/Souris River/(Point Wilkins) SOLUTION CHIMNEY (?) | 0.0-0.4 0.4-0.5 0.5-8.3 | Glacial till Rind (?): dark brown dolomitic limestone to limestone; sideritic Micritic Limestone Beds: limestone; fossiliferous; mudstone to wackestone; scattered grey clay |
| M-8-97 Dawson Bay Borrow Pit E. of Mafeking Quarry 55° angle | 12-33-44-25W 5855935N 362619E 276 m | DEVONIAN/Souris River/(Point Wilkins) | 0.0-0.2 0.2-0.3 0.3-2.0 2.0-2.2 2.2-3.5 | Dolomitic Limestone Beds: limestone; light brown to brown Rind: sideritic limestone; dark brown Micritic Limestone Beds: light brown tan limestone; mudstone Pebble infill: carbonate and Precambrian fragments Micritic Limestone Beds: (as above) |
| M-9-97 Dawson Bay Tower Outcrop | 2-8-45-25W 5858490N 361895E 274 m | DEVONIAN/Souris River/(Point Wilkins) (First Red Beds) Dawson Bay (Upper) Dawson Bay (Middle) Dawson Bay (Lower) (Second Red Beds) | 0.0-17.0 17.0-26.0 26.0-40.0 40.0-43.2 43.2-59.9 59.9-67.8 67.8-72.3 | Micritic Limestone Beds: light tan limestone; fossiliferous; mudstone Argillaceous Limestone Beds: tan to brown limestone; mudstone; abundant red colouration, fossiliferous Interbedded green and red mudstone; calcareous and non-calcareous Limestone with some dolomite; wackestone; scattered sulphide bonding at top Grey to red calcareous shale/mudstone; fossiliferous Limestone: light green to grey to red; mudstone at the base with wackestone/packstone at the top; fossiliferous Dark red to grey argillaceous dolomite; mudstone |
| M-10-97 Dawson Bay East Steeprock Bridge Pelican Rapids Road | 15-12-44-25W 5849594N 368079E 263 m | DEVONIAN/Dawson Bay (Lower) (Second Red Beds) Winnipegosis/(Lower) | 0.0-4.7 4.7-6.2 6.2-15.5 15.5-17.6 17.6-28.8 | Limestone: fossiliferous, mudstone Transitional Beds: olive grey mudstone Grey mudstone; dolomitic Transitional Beds: blue green calcareous dolomite Light brown tan packstone; porous; reefal |
| M-11-97 Dawson Bay Highway 10 outcrop Borrow Pit | 10-5-45-25W 5857289N 362112E 282 m | DEVONIAN/Dawson Bay (Upper) Dawson Bay/(Middle) Dawson Bay (Lower) (Second Red Beds) | 0.0-7.3 7.3-26.7 26.7-30.9 30.9-34.8 34.8-40.1 | Limestone; wackestone; light brown tan; pyrite blebs throughout; sulphide banding at top Limestone: dark grey; mudstone; fossiliferous Limestone: light tan brown; mudstone; scattered sulphides Transitional Beds: dolomite; mudstone; brecciated Dolomite: dark red brown mudstone |
| M-12-97 Dawson Bay Pelican Rapids Rd. & Hwy. 10 | 4-21-44-25W 5851850N 362550E 283.5 m | DEVONIAN/Souris River/(Point Wilkins) (First Red Beds) Dawson Bay/(Upper) Dawson Bay/(Middle) Dawson Bay (Lower) (Second Red Beds) | 0.0-1.8 1.8-24.1 24.1-31.5 31.5-42.0 42.0-48.3 48.3-56.2 56.2-67.2 67.2-74.5 74.5-78.3 | Dolomitic Limestone Beds: oxide cap; dark orange brown Micritic Limestone Beds: limestone; wackestone; very broken core Argillaceous Limestone Beds: limestone; mudstone; light green with some red Limestone: dolomitic; oxidized and reduced intervals Transitional Beds: limestone and dolomitic limestone, green grey argillaceous beds Limestone: wackestone; light brown tan Limestone: olive green to light tan; mudstone; fossiliferous Limestone: mudstone and wackestone; light tan to brown; some green argillaceous partings Dark red dolomitic mudstone |
| M-13-97 Dawson Bay West of Swan Lake | 2-30-41-24W 5823840N 370600E 269 m | OVERBURDEN DEVONIAN/Souris River/(Point Wilkins) INFILL or SOLUTION CHIMNEY (?) CRETACEOUS or JURASSIC? | 0.0-20.7 20.7-20.9 20.9-21.0 21.0-23.6 23.6-26.7 | Glacial till and Lake Agassiz clay(?) Micritic Limestone Beds: calcareous dolomite; mudstone; light tan to orange - looks like rind; dark brown to olive green mudstone; dark brown to olive green mudstone/clay with muscovite flakes(?) Siliceous sinter boulder (white) Till-like material with clay (?) Clay: blue green/green grey to olive green; not sure if part of solution chimney or channel infill material |
| M-14-97 Dawson Bay North Mafeking Quarry | 7-32-44-25W 585575N 361825E 278 m | DEVONIAN/Souris River/(Point Wilkins) SOLUTION CHIMNEY (?) | 0.0-5.0 5.0-5.5 | Micritic Limestone Beds: light brown tan; wackestone; poor core recovery; 5 cm of silica sinter at base Blue green to green grey clay with “micro sinters” |